## LOW VISIBILTY TAKE OFF - LVTO

- LOW VISIBILITY TAKE OFF – LVTO

  - Introduction

Take- off operations in LVTO conditions are available for RWY’s 09 and 27

The fixed minimum required RVR value for LVTO at Tuzla airport is: 125 M

  - Criteria for the implementation LVTO

- Low-visibility operations (LVO) refer to an approach to or take-off from the runway when the horizontal visibility along the runway (Runway Visual Range – RVR) is less than 550 m, or when the Decision Height (DH) is less than 200 ft.

- Low-visibility procedures (LVP) refer to the procedures applied at the aerodrome to ensure safety during low-visibility operations.

- Low-visibility take-off (LVTO) refers to a take-off when the Runway Visual Range (RVR) is less than 550 m.

  - Details for runway entries

Entry to RWY 09 is permitted only via taxiway A (ALFA)

Entry to RWY 27 is permitted only via taxiway E (ECHO)

  - Details of holding points to be used

The holding position to be used for RWY 09 is Holding position on taxiway A

( ALFA )

The holding position to be used for RWY 27 is Holding position on taxiway E (ECHO)

  - Ground movement restrictions

Operations of vehicles on the maneuvering area are to be restricted to minimum when LVTO is in progress. Aircraft movements on the apron must only be carried out according to the directions of the marshaller. Only one ACFT movement on the apron at the time is allowed when LVTO is in place (including ACFT in push-back procedure).

| SYSTEM CONSIDERED | FAILURE TO BE REPORTED | EFFECT ON FLIGHT OPERATIONS |
| --- | --- | --- |
| RVR | Touchdown RVR system for RWY 27 unserviceable | Take-off from RWY 27 is not permitted for visibility between 125 m and 300 m. Take-off from RWY 09 is permitted for visibility of 125 m or more. Take-off from RWY 27 is permitted for visibility of 300 m or more, based on pilot assessment. |
|  | Touchdown RVR system for RWY 09 unserviceable | Restrictions in Accordance with Flight Operations Rules Take-off from RWY 09 is not permitted for visibility between 125 m and 300 m. Take-off from RWY 27 is permitted for visibility of 125 m or more. Take-off from RWY 09 is permitted for visibility of 300 m or more, based on pilot assessment. |
|  | Other RVR systems a | Restrictions in Accordance with Flight Operations Rules |
| LIGHTING | Runway Lighting unserviceable(Runway End) | LVTO not permitted |
|  | Centre line RWY unserviceable | Restrictions in Accordance with Flight Operations Rules. Take-off permitted for RVR 300 m or more. |
|  | Runway edge lighting system unserviceable | LVTO not permitted |
|  | Edge Taxiway lighting system unserviceable | No effect. Use of Follow-me vehicle is mandatory. |
|  | Vertical signals unserviceable | No effect. Use of Follow-me vehicle is mandatory. |
|  | RGL unserviceable | No effect. Use of Follow-me vehicle is mandatory. |
|  | Secondary Power Supply unserviceable | LVTO not permitted |

  - Procedure in the Event of Degradation of the Runway Centre line Lighting  System

Low-visibility take-off (LVTO) under conditions of degradation of the runway centreline lighting system is conducted exclusively when the RVR is less than 550 m and greater than 300 m.

Therefore, the minimum permitted RVR value for take-off from Tuzla Airport in the event of degradation of the runway centerline lighting system, based on information provided by air traffic controllers, is 300 m.

In this case, the horizontal visibility value measured by RVR for the initial phase of the aircraft take-off operation may be replaced by a pilot assessment upon entering the take-off position on the runway.

The pilot-in-command has the option, if the measured horizontal visibility is less than 300 m but not less than 200 m, with a tendency to increase, or if the pilot expresses doubt regarding the RVR report for horizontal visibility for the initial phase of take-off, to request entry to the line-up position (take-off position) on runway 09 or 27 with a follow-me vehicle or with the aircraft, and to perform a visual assessment of horizontal visibility.

The line-up position on runway 09 from which the horizontal visibility assessment is performed is an imaginary line connecting the third runway edge light counted from the runway threshold on both the left and right sides. For runway 27, the line-up position is an imaginary line connecting the second runway edge light on the right side and the third runway edge light on the left side.

From the specified line-up positions, the pilot-in-command assesses the horizontal visibility by counting the runway edge lights on the left side of the runway. If the number of observed runway edge lights is five or more, the aircraft will be cleared for take-off from the runway on which the visual assessment of horizontal visibility was carried out.

The horizontal visibility value of 300 m represents the minimum value for aircraft take-off at Tuzla Airport for runways 09 and 27 in the event of degradation of the runway centerline lighting system, based on measured RVR values or pilot assessment. Below this minimum, operations are not permitted in such cases.